Sri Lanka Institute of Information Technology

Faculty of Computing, Department of Computer Systems Engineering

IE3122: Network Security

Week 10 Lab Guide: Local, TACACS+, and RADIUS AAA

| Field | Detail |
| --- | --- |
| Based on | Week 10 lecture (Local vs Server-Based AAA, RADIUS vs TACACS+, AAA method lists, Cisco AAA configuration) |
| Runs in | This week's lab session |
| Learning Outcomes | LO4 (evaluate security technologies), LO5 (configure security technologies) |
| Environment | Cisco Packet Tracer |
| Structure | Part 1 builds local AAA as a baseline on both routers. Part 2 adds TACACS+ on R1. Part 3 adds RADIUS on R2. Part 4 deliberately breaks each server to watch the local fallback actually happen. |
| Submission | Completed answer sheet + screenshots of the verification commands and login tests, submission method per course page |

One important note before you start. This week's lecture taught the modern Cisco IOS syntax for defining a TACACS+ or RADIUS server, tacacs server NAME and radius server NAME. Packet Tracer's simulated IOS does not reliably support that newer syntax, even in recent versions. This lab uses the older tacacs-server host and radius-server host commands instead, which Packet Tracer does support correctly. Both are genuine, real IOS commands; the newer syntax is simply what you'd actually use on real, current equipment, while the older syntax is what this simulator needs.

# Part 1, Local AAA as a Baseline

## Topology

| Device | Role |
| --- | --- |
| R1 | Router administered via TACACS+, with a local fallback account |
| TACACS-Server | A Packet Tracer Server device running the AAA/TACACS+ service, backing R1 |
| Admin-PC1 | Represents a network administrator logging into R1 to test authentication |
| R2 | Router administered via RADIUS, with a local fallback account |
| RADIUS-Server | A Packet Tracer Server device running the AAA/RADIUS service, backing R2 |
| Admin-PC2 | Represents a network administrator logging into R2 to test authentication |

Physical layout: R1 and R2 are two independent, unconnected setups, each a router with a server and an admin PC on the same LAN. You are building and testing each one separately.

## Addressing Plan

| Device | Network | Address |
| --- | --- | --- |
| R1 Gi0/0 | 192.168.1.0/24 | 192.168.1.1 |
| TACACS-Server | 192.168.1.0/24 | 192.168.1.2, gateway 192.168.1.1 |
| Admin-PC1 | 192.168.1.0/24 | 192.168.1.10, gateway 192.168.1.1 |
| R2 Gi0/0 | 192.168.2.0/24 | 192.168.2.1 |
| RADIUS-Server | 192.168.2.0/24 | 192.168.2.2, gateway 192.168.2.1 |
| Admin-PC2 | 192.168.2.0/24 | 192.168.2.10, gateway 192.168.2.1 |

## Step 1.1, Configure R1 and R2 Interfaces

R1: interface gigabitEthernet 0/0 / ip address 192.168.1.1 255.255.255.0 / no shutdown

R2: interface gigabitEthernet 0/0 / ip address 192.168.2.1 255.255.255.0 / no shutdown

Assign the TACACS-Server, RADIUS-Server, Admin-PC1, and Admin-PC2 their addresses from the plan above.

## Step 1.2, Enable AAA and Configure Local Authentication

On both R1 and R2:

enable

configure terminal

username localadmin privilege 15 secret LocalPa55

aaa new-model

aaa authentication login default local

line vty 0 4

login authentication default

transport input telnet

exit

This local account, localadmin, is your safety net for the rest of this lab. Keep its credentials in mind, you will rely on it directly in Part 4.

## Step 1.3, Confirm Local Login Works

From Admin-PC1, Telnet to 192.168.1.1 and log in as localadmin. Repeat from Admin-PC2 to 192.168.2.1.

Screenshot both successful Telnet logins using the local account.

# Part 2, Add TACACS+ on R1

## Step 2.1, Configure the TACACS+ Service on TACACS-Server

- Open TACACS-Server, go to its Services tab, and enable the AAA service.

- Under Network Configuration, add a client entry for R1: IP address 192.168.1.1, and a shared secret key of your choosing, for example tacacspa55.

- Under User Setup, add a user account, for example username tacadmin, password TacacsPa55.

## Step 2.2, Configure R1 to Use the TACACS+ Server

configure terminal

tacacs-server host 192.168.1.2

tacacs-server key tacacspa55

aaa authentication login default group tacacs+ local

This is the older syntax Packet Tracer requires. Notice the method list itself: group tacacs+ local. TACACS+ is tried first; local is the fallback if the server doesn't respond, not if it simply rejects the login.

## Step 2.3, Confirm TACACS+ Login Works

From Admin-PC1, Telnet to R1 again, this time logging in with the TACACS+ account, tacadmin.

Screenshot the successful Telnet login using the TACACS+ account.

# Part 3, Add RADIUS on R2

## Step 3.1, Configure the RADIUS Service on RADIUS-Server

- Open RADIUS-Server, go to its Services tab, and enable the AAA service.

- Under Network Configuration, add a client entry for R2: IP address 192.168.2.1, and a shared secret key of your choosing, for example radiuspa55.

- Under User Setup, add a user account, for example username radadmin, password RadiusPa55.

## Step 3.2, Configure R2 to Use the RADIUS Server

configure terminal

radius-server host 192.168.2.2

radius-server key radiuspa55

aaa authentication login default group radius local

## Step 3.3, Confirm RADIUS Login Works

From Admin-PC2, Telnet to R2 again, this time logging in with the RADIUS account, radadmin.

Screenshot the successful Telnet login using the RADIUS account.

# Part 4, Watch the Fallback Actually Happen

The method lists you configured in Parts 2 and 3 both end in local, exactly the fallback behaviour discussed in lecture. This part makes that concrete instead of theoretical: you will deliberately take each AAA server offline and prove the router still lets a legitimate administrator in.

## Step 4.1, Break TACACS+ and Retest R1

- Power off TACACS-Server, or disconnect its cable from the switch.

- From Admin-PC1, attempt to Telnet to R1 using the TACACS+ account, tacadmin. This should now fail or hang, since the server can no longer respond.

- Immediately try again, this time logging in as localadmin instead.

Screenshot the failed TACACS+ login attempt and the successful local fallback login, from the same session.

Reconnect or power TACACS-Server back on before moving to the reflection below.

## Step 4.2, Break RADIUS and Retest R2

- Power off RADIUS-Server, or disconnect its cable from the switch.

- From Admin-PC2, attempt to Telnet to R2 using the RADIUS account, radadmin. This should now fail or hang.

- Immediately try again, logging in as localadmin instead.

Screenshot the failed RADIUS login attempt and the successful local fallback login, from the same session.

Reconnect or power RADIUS-Server back on afterward.

# Reflection

Answer in a short paragraph (5 to 7 sentences): in Part 4, the router did not simply lock everyone out the moment its AAA server disappeared. Explain, using your own results, exactly why the fallback worked, referring specifically to what the method list command actually says. Then explain what would have happened instead if the method list had been written as just group tacacs+ (or group radius), with no local at the end, and why an organisation would need to weigh that risk deliberately rather than assume server-based AAA is always safe to rely on alone.