Sri Lanka Institute of Information Technology

Faculty of Computing, Department of Computer Systems Engineering

IE3122: Network Security

Week 5 Lab Guide: Routing Protocol Hardening

| Field | Detail |
| --- | --- |
| Based on | Week 5, Routing Protocols and Routing Attacks lecture (protocol fundamentals, routing attacks, defenses) |
| Runs in | Week 6 lab session (labs trail lectures by one week) |
| Learning Outcomes | LO2 (evaluate mitigation techniques) and LO5 (configure security technologies) |
| Environment | Cisco Packet Tracer |
| Structure | Section 1: build the routed topology. Section 2: configure, harden, and test OSPF, using a copy of the base file. Section 3: configure, harden, and test EIGRP, using a second copy of the base file. |
| Submission | Completed answer sheet + screenshots of the verification commands, submission method per course page |

You'll build the base topology once in Section 1, then save two separate copies of it, one for each routing protocol. That way Sections 2 and 3 don't interfere with each other, and you keep both working, hardened topologies at the end instead of overwriting one with the other. Each protocol gets the same treatment: configure it, verify it works, harden it with MD5 authentication, then prove the hardening actually does something before moving on.

# Section 1, Build the Routed Architecture

## Topology

| Device | Role |
| --- | --- |
| R1, R2, R3 | Routers in a simple line topology: R1 to R2 to R3 |
| PC1, PC2, PC3 | One end host per router, used to test end-to-end connectivity across the whole path |

Physical layout: PC1 to R1 to R2 to R3 to PC3, with PC2 also attached directly to R2. Build this in Packet Tracer before configuring anything.

## Port and Addressing Plan

| Router | Interface | Connects to | Address |
| --- | --- | --- | --- |
| R1 | GigabitEthernet0/0 | PC1 (LAN) | 192.168.1.1 /24 |
| R1 | GigabitEthernet0/1 | R2 | 10.0.12.1 /30 |
| R2 | GigabitEthernet0/0 | PC2 (LAN) | 192.168.2.1 /24 |
| R2 | GigabitEthernet0/1 | R1 | 10.0.12.2 /30 |
| R2 | GigabitEthernet0/2 | R3 | 10.0.23.1 /30 |
| R3 | GigabitEthernet0/0 | PC3 (LAN) | 192.168.3.1 /24 |
| R3 | GigabitEthernet0/1 | R2 | 10.0.23.2 /30 |

| PC | IP Address | Default Gateway |
| --- | --- | --- |
| PC1 | 192.168.1.10 /24 | 192.168.1.1 |
| PC2 | 192.168.2.10 /24 | 192.168.2.1 |
| PC3 | 192.168.3.10 /24 | 192.168.3.1 |

## Step 1.1, Configure Router Interfaces

On each router, assign the addresses from the plan above and bring the interfaces up. Example for R1:

enable

configure terminal

interface gigabitEthernet 0/0

ip address 192.168.1.1 255.255.255.0

no shutdown

exit

interface gigabitEthernet 0/1

ip address 10.0.12.1 255.255.255.252

no shutdown

exit

Repeat the equivalent configuration on R2 (both its GigabitEthernet0/0 LAN interface and its two point-to-point interfaces) and R3, using their own addresses from the plan.

## Step 1.2, Configure the PCs

Assign each PC its static IP address and default gateway from the plan above.

## Step 1.3, Verify Before Adding Routing

At this point, no routing protocol is running yet. Confirm the following, and think about why each result makes sense:

- show ip interface brief on each router: every interface should show up/up.

- PC1 can ping its own gateway (192.168.1.1), but cannot ping PC2 or PC3 yet.

- R1 can ping R2 directly (10.0.12.2), since they're on a shared link, but cannot ping R3 or PC3.

This is exactly the problem routing protocols solve, per this week's lecture: each router only knows its own directly connected networks until something builds the rest of the routing table for it.

Screenshot show ip interface brief from all three routers, and the failed PC1-to-PC3 ping, as your baseline evidence.

## Step 1.4, Save and Duplicate the File

Save your Packet Tracer file now, before configuring any routing protocol. Then make two copies of it:

- Rename or save-as one copy something like YourName_Week5_OSPF.pkt. Use this copy for Section 2.

- Rename or save-as a second copy something like YourName_Week5_EIGRP.pkt. Use this copy for Section 3.

Working from two separate copies means Sections 2 and 3 never interfere with each other, and you'll finish the lab with two complete, independently hardened topologies to submit, not just one.

# Section 2, Configure, Harden, and Test OSPF

## Step 2.1, Configure OSPF

On all three routers, enable OSPF process 1 and advertise every directly connected network into area 0. Example for R1:

configure terminal

router ospf 1

network 192.168.1.0 0.0.0.255 area 0

network 10.0.12.0 0.0.0.3 area 0

exit

Repeat on R2 (all three of its networks) and R3 (both of its networks), each still under area 0.

## Step 2.2, Verify OSPF Is Working

show ip ospf neighbor

show ip route ospf

Confirm every router shows the expected neighbor(s) in the FULL state, and that each router's routing table now contains OSPF-learned routes to the networks it isn't directly connected to.

Test end-to-end: ping PC3 from PC1. It should now succeed.

Screenshot show ip ospf neighbor from R2 (it should show two neighbors) and the successful PC1-to-PC3 ping.

## Step 2.3, Harden with MD5 Authentication

Enable MD5 authentication for area 0 on all three routers, then set a matching key on every interface participating in OSPF. Example for R1:

configure terminal

router ospf 1

area 0 authentication message-digest

exit

interface gigabitEthernet 0/1

ip ospf message-digest-key 1 md5 OspfKey5

exit

Repeat the area authentication command on R2 and R3, and set the same key (OspfKey5, key ID 1) on every OSPF-participating interface on all three routers, including R2's two point-to-point interfaces.

## Step 2.4, Verify the Hardening Took Effect

show ip ospf interface gigabitEthernet 0/1

show ip ospf neighbor

Confirm the interface output shows message-digest authentication enabled, and that the neighbor relationships are still FULL now that every router shares the same key.

Screenshot show ip ospf interface for one interface, showing authentication enabled.

## Step 2.5, Prove the Authentication Actually Works

Temporarily change the key on one interface only, for example R2's link toward R3, to a different value:

configure terminal

interface gigabitEthernet 0/2

ip ospf message-digest-key 1 md5 WrongKey

exit

Wait a short moment, then check the neighbor relationship on that link:

show ip ospf neighbor

The R2 to R3 neighbor relationship should now be down or missing entirely. That's the authentication doing its job: R2 and R3 no longer agree on the key, so they refuse to trust each other's routing updates.

Screenshot the broken neighbor relationship. Then restore the correct key (OspfKey5) on that interface, confirm the neighbor relationship comes back, and screenshot that too.

# Section 3, Configure, Harden, and Test EIGRP

Open your second copy of the base topology, the one you haven't touched since Step 1.4. It should still show no routing protocol configured and the same connectivity gap you confirmed back in Step 1.3. You'll now repeat the same configure, verify, harden, verify, and prove-it-works sequence, this time for EIGRP.

## Step 3.1, Configure EIGRP

On all three routers, enable EIGRP autonomous system 10 and advertise every directly connected network. Example for R1:

configure terminal

router eigrp 10

network 192.168.1.0 0.0.0.255

network 10.0.12.0 0.0.0.3

exit

Repeat on R2 and R3 with their own networks, using the same AS number, 10, on every router.

## Step 3.2, Verify EIGRP Is Working

show ip eigrp neighbors

show ip route eigrp

Confirm every router shows the expected neighbor(s), and that routing tables now contain EIGRP-learned routes. Test end-to-end: ping PC3 from PC1 again.

Screenshot show ip eigrp neighbors from R2 and the successful PC1-to-PC3 ping.

## Step 3.3, Harden with MD5 Authentication

Unlike OSPF, EIGRP authentication needs a key chain defined first. Example for R1:

configure terminal

key chain EIGRPCHAIN

key 1

key-string EigrpKey5

exit

exit

interface gigabitEthernet 0/1

ip authentication mode eigrp 10 md5

ip authentication key-chain eigrp 10 EIGRPCHAIN

exit

Repeat the full key chain definition and both interface authentication commands on R2 (both its interfaces) and R3, using the same key chain name, key ID, and key-string everywhere.

## Step 3.4, Verify the Hardening Took Effect

show key chain

show ip eigrp neighbors

Confirm the key chain is correctly defined, and that neighbor relationships are still up now that every router shares the same key.

Screenshot show key chain from one router, and show ip eigrp neighbors showing the adjacency still up.

## Step 3.5, Prove the Authentication Actually Works

Temporarily change the key-string in R2's key chain to a different value:

configure terminal

key chain EIGRPCHAIN

key 1

key-string WrongKey

exit

exit

Check the neighbor relationships:

show ip eigrp neighbors

R2's neighbor relationships should now drop, since it no longer agrees with R1 or R3 on the key.

Screenshot the dropped neighbor relationship(s). Then restore the correct key-string (EigrpKey5), confirm the neighbors come back, and screenshot that too.

# Reflection

Answer in a short paragraph (4 to 6 sentences): comparing Sections 2 and 3, which protocol's authentication was more work to configure, and did that extra effort (if any) actually buy you anything, referring back to the OSPF-versus-EIGRP comparison from this week's lecture?