Sri Lanka Institute of Information Technology

Faculty of Computing, Department of Computer Systems Engineering

IE3122: Network Security

Week 7 Lab Guide: A DMZ and Zone-Based Policy on a Single Firewall

| Field | Detail |
| --- | --- |
| Based on | Week 7 Deck B, Firewall Placement and ACLs lecture (the DMZ, Zone-Based Firewall as a configuration model, and ACL configuration) |
| Runs in | This week's lab session |
| Learning Outcomes | LO3 (analyse security requirements), LO4 (evaluate security technologies), LO5 (configure security technologies) |
| Environment | Cisco Packet Tracer |
| Structure | Part 1 builds the topology, one router with three zones and no routing to configure at all, since every network is directly connected. Parts 2 to 4 configure one Extended ACL per interface, together behaving like a simple zone-based packet-filtering firewall. |
| Submission | Completed answer sheet + screenshots of the verification commands, submission method per course page |

Packet Tracer has no dedicated Zone-Based Firewall device, and this module hasn't covered ZBF's own zone/zone-pair CLI syntax. This lab gets the same practical result a different way: one Extended ACL per interface on a single router, each one enforcing the policy for a single zone. The DMZ in this lab hosts two different services with two different exposure levels, and Inside has one ordinary user and one administrator with different privileges, specifically so you can see that a zone-based design is genuinely flexible, not just a blunt all-or-nothing split between three networks.

# Part 1, Build the Topology and Confirm Baseline Access

## Topology

| Device | Zone | Role |
| --- | --- | --- |
| Outside-PC | Outside | Represents an ordinary external internet user |
| Outside-Server | Outside | Represents an external website that Inside staff need to browse to |
| Partner-PC | Outside | Represents one specific trusted business partner, who should reach the FTP server and nothing else |
| Firewall-Router | N/A, is the firewall | A single router with one Access Control List per interface. This is the entire firewall in this lab |
| Web-Server | DMZ | Public-facing web service, reachable by anyone over HTTP |
| FTP-Server | DMZ | Partner-facing file service, reachable only from Partner-PC, not the general public |
| PC-Inside | Inside | An ordinary internal user |
| PC-Inside-Admin | Inside | The one internal host allowed to manage the DMZ web server directly |

Physical layout: a single router, Firewall-Router, with three interfaces. Gi0/0 connects to a switch carrying Outside-PC, Outside-Server, and Partner-PC. Gi0/1 connects to a DMZ switch carrying Web-Server and FTP-Server. Gi0/2 connects to an Inside switch carrying PC-Inside and PC-Inside-Admin. Every network here is directly connected to Firewall-Router, so unlike a design with a separate border router in front of it, no static routing is needed anywhere in this lab.

## Addressing Plan

| Device/Interface | Network | Address |
| --- | --- | --- |
| Outside-PC | 203.0.113.0/29 | 203.0.113.2, gateway 203.0.113.1 |
| Outside-Server | 203.0.113.0/29 | 203.0.113.3, gateway 203.0.113.1 |
| Partner-PC | 203.0.113.0/29 | 203.0.113.4, gateway 203.0.113.1 |
| Firewall-Router Gi0/0 (Outside zone) | 203.0.113.0/29 | 203.0.113.1 |
| Firewall-Router Gi0/1 (DMZ zone) | 192.168.100.0/24 | 192.168.100.1 |
| Firewall-Router Gi0/2 (Inside zone) | 192.168.200.0/24 | 192.168.200.1 |
| Web-Server (DMZ) | 192.168.100.0/24 | 192.168.100.10, gateway 192.168.100.1 |
| FTP-Server (DMZ) | 192.168.100.0/24 | 192.168.100.11, gateway 192.168.100.1 |
| PC-Inside | 192.168.200.0/24 | 192.168.200.10, gateway 192.168.200.1 |
| PC-Inside-Admin | 192.168.200.0/24 | 192.168.200.11, gateway 192.168.200.1 |

## Step 1.1, Configure Firewall-Router

enable

configure terminal

interface gigabitEthernet 0/0

ip address 203.0.113.1 255.255.255.248

no shutdown

exit

interface gigabitEthernet 0/1

ip address 192.168.100.1 255.255.255.0

no shutdown

exit

interface gigabitEthernet 0/2

ip address 192.168.200.1 255.255.255.0

no shutdown

exit

No ip route command appears anywhere in this lab. All three networks sit on interfaces of the same router, so all three are directly connected as far as Firewall-Router is concerned, and it already knows how to reach every one of them the moment each interface comes up.

## Step 1.2, Configure the End Devices

Assign every PC and server its static address and gateway from the plan above. On Outside-Server, enable the HTTP service. On Web-Server, enable both the HTTP service and the Telnet (remote access) service, with a password set for Telnet. On FTP-Server, enable the FTP service with a username and password of your choosing.

## Step 1.3, Confirm Baseline Access Before Any ACL Exists

With no ACL configured anywhere yet, confirm every one of these succeeds:

- Outside-PC can ping both Web-Server and FTP-Server, and can open Web-Server's web page.

- Partner-PC can reach FTP-Server's FTP service.

- Outside-PC can ping both PC-Inside and PC-Inside-Admin.

- PC-Inside and PC-Inside-Admin can both ping Outside-Server and both Telnet to Web-Server.

- Web-Server and FTP-Server can both ping PC-Inside.

This confirms the topology genuinely works end to end before any security policy exists. If any of these fail now, fix that first.

Screenshot a successful ping from Outside-PC to Web-Server, and a successful ping from Web-Server to PC-Inside.

# Part 2, The Outside Zone: Two Services, Two Different Exposure Levels

Policy for this zone: the general public may reach the web server over HTTP. Only the named partner may reach the FTP server, over FTP. Nobody from outside may reach anything else at all, including the Inside network.

## Step 2.1, Configure and Apply the Outside ACL

configure terminal

access-list 101 permit tcp any host 192.168.100.10 eq 80

access-list 101 permit tcp host 203.0.113.4 host 192.168.100.11 eq 21

access-list 101 permit tcp any any established

access-list 101 deny ip any any

interface gigabitEthernet 0/0

ip access-group 101 in

exit

Notice the second line names Partner-PC specifically as the source, not any. This is exactly what a DMZ makes possible that a single flat network doesn't: two services in the same zone, exposed to two completely different audiences, controlled by two separate lines in the same ACL. The third line, permitting established TCP traffic, is what lets replies to connections Inside or the DMZ started get back in; without it, this ACL would also silently block Inside's and the DMZ's own outbound traffic.

## Step 2.2, Verify

show access-lists 101

- Confirm Outside-PC can still open Web-Server's web page.

- Confirm Outside-PC cannot reach FTP-Server's FTP service at all.

- Confirm Partner-PC can reach FTP-Server's FTP service.

- Confirm neither Outside-PC nor Partner-PC can ping Web-Server, FTP-Server, or anything on the Inside network.

Screenshot Outside-PC's successful web page load and failed FTP attempt, and Partner-PC's successful FTP connection.

# Part 3, The DMZ Zone: One Rule Protecting Every Host In It

Policy for this zone: if either DMZ server is ever compromised, an attacker sitting on it must not be able to reach the Inside network. Both servers may still reach out to the internet.

## Step 3.1, Configure and Apply the DMZ ACL

configure terminal

access-list 102 deny ip 192.168.100.0 0.0.0.255 192.168.200.0 0.0.0.255

access-list 102 permit ip any any

interface gigabitEthernet 0/1

ip access-group 102 in

exit

## Step 3.2, Verify, Simulating a Compromise of Each Server in Turn

Imagine each DMZ server in turn has just been compromised by an attacker, and that attacker is now trying to move sideways into the Inside network.

show access-lists 102

- Confirm Web-Server can no longer ping PC-Inside or PC-Inside-Admin.

- Confirm FTP-Server can no longer ping PC-Inside or PC-Inside-Admin either.

- Confirm both Web-Server and FTP-Server can still ping out to Outside-PC or Outside-Server.

Screenshot the failed ping from each DMZ server to PC-Inside, and one successful ping from a DMZ server outward.

One line of policy just protected two different servers, and would protect a third or a fourth added to this same DMZ later without any changes at all. This is the real architectural advantage of a DMZ: the containment guarantee comes from where a host sits, not from configuration you have to remember to repeat for every service you add to it.

# Part 4, The Inside Zone: Trusted Does Not Have to Mean Unrestricted

Policy for this zone: PC-Inside-Admin may Telnet into Web-Server to manage it. No other Inside host may. Every other kind of traffic from Inside is allowed to leave freely.

## Step 4.1, Configure and Apply the Inside ACL

configure terminal

access-list 103 permit tcp host 192.168.200.11 host 192.168.100.10 eq 23

access-list 103 deny tcp any host 192.168.100.10 eq 23

access-list 103 permit ip any any

interface gigabitEthernet 0/2

ip access-group 103 in

exit

Order matters here more than anywhere else in this lab. The specific permit for PC-Inside-Admin must come first. If the general deny came first instead, it would catch PC-Inside-Admin's own Telnet attempt too, since deny lines don't know or care which host they're written to stop unless a more specific line already matched above them.

## Step 4.2, Run the Full Verification Matrix

With all three ACLs in place, confirm every one of the following matches the policy you've built:

| Test | Expected Result |
| --- | --- |
| Outside-PC to Web-Server, HTTP | Succeeds |
| Outside-PC to FTP-Server, FTP | Fails |
| Partner-PC to FTP-Server, FTP | Succeeds |
| Outside-PC to PC-Inside, ping | Fails |
| Web-Server to PC-Inside, ping | Fails |
| FTP-Server to PC-Inside, ping | Fails |
| Web-Server or FTP-Server to Outside-PC, ping | Succeeds |
| PC-Inside-Admin to Web-Server, Telnet | Succeeds |
| PC-Inside to Web-Server, Telnet | Fails |
| PC-Inside to Outside-Server, HTTP | Succeeds |
| PC-Inside to Outside-PC or Outside-Server, ping | Fails, see the note below |

The last row is the one worth understanding, not just accepting. PC-Inside's ping leaves freely, but the reply is ICMP, and the established keyword in Part 2 only recognises TCP connections. ICMP has no such concept at all. The reply arrives back on the Outside interface, matches nothing in that ACL except the final deny, and is dropped. HTTP succeeds outward across the same three zones because TCP's established keyword covers it; ping does not, because ICMP was never designed to have an established state in the first place.

Screenshot the results of all eleven tests in the table above, including the failed Telnet attempt from PC-Inside and the failed ping from PC-Inside outward.

# Reflection

Answer in a short paragraph (5 to 7 sentences): you never configured a single zone, zone-pair, or anything called a firewall in this lab, only three ordinary Extended ACLs. Explain what specifically about how they were designed, one per interface, each enforcing that interface's own zone policy, makes the end result functionally equivalent to a simple zone-based packet-filtering firewall. Then choose one of the two ‘exceptions within a zone’ you configured, either the FTP server's partner-only access or PC-Inside-Admin's Telnet privilege, and explain what real-world problem that specific design solves that a simpler policy of ‘the whole zone is either open or closed’ would not.