Sri Lanka Institute of Information Technology

Faculty of Computing, Department of Computer Systems Engineering

IE3122: Network Security

Week 9 Lab Guide: GRE Tunnels and a Site-to-Site IPsec VPN

| Field | Detail |
| --- | --- |
| Based on | Week 9 lecture (tunnel vs VPN, GRE, IPsec's modular framework, Cisco VPN configuration) |
| Runs in | This week's lab session |
| Learning Outcomes | LO4 (evaluate security technologies), LO5 (configure security technologies) |
| Environment | Cisco Packet Tracer. Use 1941 routers specifically. The security license command in Part 3 is tied to that router family |
| Structure | Part 1 builds plain IP connectivity. Part 2 adds a GRE tunnel. Part 3 adds IPsec to protect that tunnel. Each part ends by inspecting the actual packet on the wire, so you see exactly what changes and what doesn't. |
| Submission | Completed answer sheet + screenshots of the verification commands and packet inspections, submission method per course page |

This lab builds the same two-site network three times over, each time adding one layer, and each time you will use Packet Tracer's Simulation Mode to actually look inside a packet as it crosses the WAN link. This is the single best way to see the difference between a tunnel and a VPN for yourself, rather than just reading about it: watch the same ping's packet change shape as you add GRE, then add IPsec on top.

# Part 1, Plain IP Connectivity, No Tunnel Yet

## Topology

| Device | Role |
| --- | --- |
| R1 | Cisco 1941 router. Connects LAN1 and the WAN link. All GRE and IPsec configuration for this side happens here |
| R2 | Cisco 1941 router. Connects LAN2 and the WAN link, mirroring R1's configuration |
| Switch1, Switch2 | Ordinary access switches, one per LAN, no special configuration needed |
| PC1 | Represents an end user on LAN1 |
| PC2 | Represents an end user on LAN2 |

Physical layout: PC1, on LAN1 behind R1, needs to reach PC2, on LAN2 behind R2. R1 and R2 connect directly to each other over a simulated WAN link.

## Addressing Plan

| Device/Interface | Network | Address |
| --- | --- | --- |
| R1 Gi0/0 (LAN1) | 192.168.10.0/24 | 192.168.10.1 |
| R1 Gi0/1 (WAN) | 203.0.113.0/30 | 203.0.113.1 |
| R2 Gi0/0 (LAN2) | 192.168.20.0/24 | 192.168.20.1 |
| R2 Gi0/1 (WAN) | 203.0.113.0/30 | 203.0.113.2 |
| PC1 | 192.168.10.0/24 | 192.168.10.10, gateway 192.168.10.1 |
| PC2 | 192.168.20.0/24 | 192.168.20.10, gateway 192.168.20.1 |

## Step 1.1, Configure R1

enable

configure terminal

interface gigabitEthernet 0/0

ip address 192.168.10.1 255.255.255.0

no shutdown

exit

interface gigabitEthernet 0/1

ip address 203.0.113.1 255.255.255.252

no shutdown

exit

## Step 1.2, Configure R2

enable

configure terminal

interface gigabitEthernet 0/0

ip address 192.168.20.1 255.255.255.0

no shutdown

exit

interface gigabitEthernet 0/1

ip address 203.0.113.2 255.255.255.252

no shutdown

exit

## Step 1.3, Configure PC1 and PC2

Assign PC1 and PC2 their static addresses and gateways from the plan above.

## Step 1.4, Add Static Routes

Neither router has a direct interface on the other's LAN, so each needs a route pointing across the WAN link:

R1: ip route 192.168.20.0 255.255.255.0 203.0.113.2

R2: ip route 192.168.10.0 255.255.255.0 203.0.113.1

## Step 1.5, Confirm Connectivity

From PC1, ping PC2. This should succeed.

Screenshot the successful ping from PC1 to PC2.

## Step 1.6, Inspect the Packet on the WAN Link

Switch Packet Tracer to Simulation Mode. Send a Simple PDU from PC1 to PC2, then step through it event by event using Capture/Forward until it appears on the link between R1 and R2.

- Click the packet on that link and open the PDU Information window.

- Open the Outbound PDU Details (or Inbound PDU Details) tab and find the IP header.

- Record the Source IP and Destination IP shown.

Record: Source = _____________  Destination = _____________

With no tunnel in place, the packet crossing the WAN link is just the original packet. Its header shows the real endpoints, PC1 and PC2, in the open, the entire way. Keep this result to compare against Parts 2 and 3.

# Part 2, Add a GRE Tunnel

A GRE tunnel wraps the original packet inside a new one. It adds no security at all, but it does change what the WAN link actually carries, and that change is exactly what you're about to see.

## Step 2.1, Configure the Tunnel Interface on R1

configure terminal

interface Tunnel0

ip address 172.16.0.1 255.255.255.252

tunnel source GigabitEthernet0/1

tunnel destination 203.0.113.2

exit

## Step 2.2, Configure the Tunnel Interface on R2

configure terminal

interface Tunnel0

ip address 172.16.0.2 255.255.255.252

tunnel source GigabitEthernet0/1

tunnel destination 203.0.113.1

exit

## Step 2.3, Redirect LAN-to-LAN Traffic Through the Tunnel

The tunnel exists now, but nothing uses it yet. Replace each router's static route so LAN traffic travels through the tunnel interface instead of the direct WAN link:

R1: no ip route 192.168.20.0 255.255.255.0 203.0.113.2

R1: ip route 192.168.20.0 255.255.255.0 172.16.0.2

R2: no ip route 192.168.10.0 255.255.255.0 203.0.113.1

R2: ip route 192.168.10.0 255.255.255.0 172.16.0.1

## Step 2.4, Confirm Connectivity

show interfaces tunnel 0

Confirm the tunnel line protocol is up, then from PC1, ping PC2 again. This should still succeed.

Screenshot show interfaces tunnel 0 confirming the tunnel is up, and the successful ping.

## Step 2.5, Inspect the Packet on the WAN Link Again

Repeat exactly the same inspection as Step 1.6: Simulation Mode, a Simple PDU from PC1 to PC2, captured on the link between R1 and R2.

- Open the PDU Information window and find the outermost IP header. Record its Source and Destination.

- Look one layer further in (the encapsulated GRE payload). You should find the original inner IP header still present, still showing PC1 and PC2's real addresses.

Record: Outer Source = _____________  Outer Destination = _____________

Record: Inner Source = _____________  Inner Destination = _____________

This is the tunnel concept made visible. The outer header now shows R1 and R2's own WAN addresses, not PC1 and PC2's, because GRE has wrapped the original packet inside a new one addressed router-to-router. The original packet did not disappear, it's sitting one layer in, completely readable to anyone who captures this traffic. GRE hides nothing. It only relabels the outside.

# Part 3, Secure the Tunnel with IPsec

GRE over IPsec: keep the tunnel exactly as it is, and add IPsec to actually protect what's now traveling across the WAN link. The interesting-traffic ACL in this design matches the GRE traffic itself, not the original LAN traffic, since by the time IPsec sees it, it's already GRE-encapsulated.

## Step 3.1, Enable the Security Technology Package

Packet Tracer's 1941 routers ship with crypto commands disabled until this license is enabled. Do this on both R1 and R2:

enable

configure terminal

license boot module c1900 technology-package securityk9

Accept the license agreement when prompted, then:

end

copy running-config startup-config

reload

The router must reload for this to take effect. Confirm it worked with show version afterward; securityk9 should now appear under the current technology packages.

## Step 3.2, Configure ISAKMP (Phase 1) on R1

configure terminal

crypto isakmp policy 10

encryption aes 256

authentication pre-share

group 5

exit

crypto isakmp key vpnpa55 address 203.0.113.2

Group 5 is used deliberately here rather than the Group 14 the lecture named as today's realistic production minimum. Packet Tracer's simulated crypto engine does not support Group 14; Group 5 is the highest group it actually implements.

## Step 3.3, Configure ISAKMP (Phase 1) on R2

configure terminal

crypto isakmp policy 10

encryption aes 256

authentication pre-share

group 5

exit

crypto isakmp key vpnpa55 address 203.0.113.1

## Step 3.4, Configure the Transform Set (Phase 2), Both Routers

crypto ipsec transform-set VPN-SET esp-aes esp-sha-hmac

## Step 3.5, Define Interesting Traffic

This ACL matches the GRE traffic itself, protocol 47, between the two routers' real WAN addresses:

R1: access-list 110 permit gre host 203.0.113.1 host 203.0.113.2

R2: access-list 110 permit gre host 203.0.113.2 host 203.0.113.1

## Step 3.6, Configure the Crypto Map and Apply It

On R1:

crypto map VPN-MAP 10 ipsec-isakmp

set peer 203.0.113.2

set transform-set VPN-SET

match address 110

exit

interface gigabitEthernet 0/1

crypto map VPN-MAP

exit

On R2, the mirror image:

crypto map VPN-MAP 10 ipsec-isakmp

set peer 203.0.113.1

set transform-set VPN-SET

match address 110

exit

interface gigabitEthernet 0/1

crypto map VPN-MAP

exit

The crypto map is applied to the physical WAN interface, Gi0/1, not to Tunnel0. IPsec is protecting the GRE-encapsulated traffic as it actually appears on the wire, which is exactly what the interesting-traffic ACL in Step 3.5 was written to match.

## Step 3.7, Confirm Connectivity

show crypto isakmp sa

show crypto ipsec sa

From PC1, ping PC2 once more to trigger the tunnel to actually negotiate, then re-run the two show commands above. Confirm an ISAKMP SA and an IPsec SA now exist between 203.0.113.1 and 203.0.113.2.

Screenshot show crypto isakmp sa and show crypto ipsec sa showing an active security association, and the successful ping.

## Step 3.8, Inspect the Packet on the WAN Link One Last Time

Repeat the same Simulation Mode inspection as before, on the link between R1 and R2.

- The outermost IP header's Source and Destination should be unchanged from Part 2, still R1 and R2's own WAN addresses. IPsec does not change who the visible endpoints are; it changes what's readable behind them.

- Note what protocol the outer header now reports carrying. It should no longer show GRE directly; ESP is now what's visible at that layer.

Record: Outer Source = _____________  Outer Destination = _____________  Protocol shown = _____________

Check this specific screen in your own Packet Tracer version before relying on it in front of a class. Packet Tracer is a teaching simulator, not a real packet capture tool, and different versions vary in exactly how much of an encrypted payload's structure they still display versus grey out. The concept to confirm either way: the addressing you can see is identical to Part 2, but the payload should no longer present as plain, readable GRE.

# What Changed, and What Didn't

| Part | Outer header on the WAN link | Is the original PC1/PC2 packet readable? |
| --- | --- | --- |
| 1, No tunnel | Source = PC1, Destination = PC2 | Yes, it is the only packet there is |
| 2, GRE tunnel | Source = R1 (203.0.113.1), Destination = R2 (203.0.113.2) | Yes, one layer in, in plain text |
| 3, GRE over IPsec | Source = R1 (203.0.113.1), Destination = R2 (203.0.113.2) | No, protected inside ESP |

Notice Parts 2 and 3 show identical outer addressing. This is precisely the lecture's tunnel-versus-VPN distinction, made concrete: encapsulation alone (Part 2) changes what address is visible, but changes nothing about whether the contents are actually protected. Only adding real encryption (Part 3) does that.

# Reflection

Answer in a short paragraph (5 to 7 sentences): using your own recorded results from the three packet inspections, explain in your own words why Part 2 alone would not be sufficient for an organisation sending genuinely sensitive data over the internet, even though the ping worked perfectly and the original addresses were no longer the first thing visible on the wire. Then explain what specifically Part 3 added that Part 2 did not, referencing the actual protocol change you observed at the outer header.